[image: image5.jpg]


 Learning to work safely

19/11/04

The safe use of hand tools: Technology (Mandatory)
In Technology you will learn about the correct and safe use of tools and materials for specific processes that are used to construct a project.

This material addresses aspects of the following syllabus outcome: 

LS 3.4
A student demonstrates safe practices in the use of materials, tools, and equipment in the context of producing a design project.

Extract from: Stage 4 Technology Syllabus ( Board of Studies NSW 2003.

The safe use of hand tools

Carefully read the following passage about the use of hand tools in the workshop and complete the questions.  

	· When undertaking a project in the workshop, never use any hand tool unless you have had instruction from your teacher and are familiar with the correct use of a tool. If you have a question about the use of a hand tool, ask your teacher.

	· Hands should be kept behind the cutting edge of sharp tools such as chisels and gouges when removing waste material.
	[image: image1.jpg]



This photo shows an unsafe practice.



	· Inspect tools before each use and repair or replace if worn or damaged. Keep cutting edges sharp. Never test a cutting edge with your fingers. Test the operation of the tool on a scrap piece of material.



	· Hold material securely on the bench with a bench hook, G-cramp or vice when working on it. Use light slow strokes when commencing a saw cut and ease the pressure as the saw breaks through the material being cut.
	[image: image2.jpg]



A bench hook.



	· Use light slow strokes when commencing a saw cut and ease the pressure as the saw breaks through the material being cut. 

· Ensure that material is never allowed to protrude past the edge of the bench.
	[image: image3.jpg]



Using a bench hook for sawing.

	· When carrying sharp tools, point cutting edges away from you. Never put sharp or pointed tools in your pockets.

· Where appropriate, wear personal protective equipment (PPE) such as safety glasses, safety goggles, ear protection and hair restraint.
	[image: image4.jpg]



A safe method to carry sharp tools.


Activity: Hand tool guidelines

Complete the following sentences by inserting appropriate wording:

1. Never use any hand tool unless you are familiar with its use and have had relevant __________. 

2. If you have a question about how to use a hand tool, ______ your teacher. 

3. Inspect tools before each use and _________ or replace if worn or damaged. Keep cutting edges _______.

4. Never test a cutting _______ with your fingers. Test the operation of the tool on a _______ piece of material. 

5. Hold _________ securely on the bench with a bench hook, G-cramp or vice when working on it. 

6. Use light slow strokes when commencing a saw _____ and ease the __________ as the saw breaks through the material being cut. 

7. Ensure that __________ is never allowed to protrude past the edge of the bench. 

8. Keep hands behind the ________ edge of sharp tools such as chisels and gouges when removing waste material.

9. When carrying sharp tools, point cutting _______ away from you. Never put sharp or pointed ________ in your pockets

10. Wear personal protective equipment (PPE) such as safety _______, safety goggles, ear protection and hair restraint where appropriate in the workshop.

	[image: image6.jpg]



	Technology Unit, Curriculum K–12 Directorate, 

NSW Department of Education and Training
http://www.curriculumsupport.nsw.edu.au/technology/
	LTWS03.doc

page 2 of 2



[image: image5.jpg][image: image6.jpg]